| SHIP TYPE |  | Special Service Vessel – Seismic Research |  |
| --- | --- | --- | --- |
| CLASSIFICATION |  | BUREAU VERITAS |  |
| BUILDER |  | REMONTOWA SA. Gdansk, Poland, 2012 |  |
| YEAR CONVERTED |  | 2019 |  |
| IMO NUMBER |  | 9606699 |  |
| PORT OF REGISTRY |  | Nassau |  |
| FLAG |  | Bahamas |  |
| CALL SIGN |  | C6EJ6 |  |
| OFFICIAL NUMBER |  | 7001264 |  |
| OWNER: | REM ANDES AS, 6092 FOSNAVAG | MANAGER: | REM MARITIME AS, 6092 FOSNAVAG |
| MAIN DIMENSIONS |  | CLASS NOTATIONS |  |
| Length O. A. | 87,90 m |  |  |
| Length P. P. | 81.52 m |  |  |
| Breadth | 18,80 m |  |  |
| Draft | 6,50 m |  |  |
| Gross Tonnage | 5458 |  |  |
| Net Tonnage | 1637 |  |  |

| MACHINERY AND PROPELLER PLANTS Main propulsion system; Main Engine 2 x MAC 6M25 rated 1900 kW (2548 BHP) each El Motor PTI 2 x PTI Motors rated 2000 kW (2682 BHP) each Combined Propulsion Hybrid Mode 6500 Kw/8716 BHP Propellers 2 x CPP, Berg, 3m Bow Thruster 2 x 900 kW,12,5 T Thrust, Scana. Stern Thruster 2 x 600 kW, 8.7 T Thrust, Scana. Power Plant; Main Generator 2 x CAT 3516C rated 2,350 kW (3,151 BHP) /690V/ 60Hz/ 3Ph Shaft Generator 2 x 2,000 kW (2,435 BHP) /690V/ 60Hz/ 3Ph Emergency Generator 1 x 398 kW (533 BHP)/ 690V/ 60Hz/ 3Ph. DECK MACHINERY Deck Crane: TTS GPT 80-2-12 Telescope Hydraulic Crane SWL 2T Deck Crane MELCAL FL35T5 Offshore Crane, SWL:4T/4,6m-1.3T/14m Anchor Windlass 2 x 11 T, Electric driven, Brattvaag Anchor Chain 2 x 275 m x 54 mm (Ø) Anchor (Bow) 2 x 2,840 kg, High Holding Power Anchors Capstan 2 x 10 T @ 35.2m/min, Brattvaag Workboat 1 x Jiangyin Neptune Marine FRP Static Bollard Pull 101.90 MT NAVIGATION EQUIPMENT Auto Pilot: SPERRY MARINE “NAVPILOT 4000” DGPS: FURUNO GP-150 Radar s: FURUNO FAR-2X17B FURUNO FAR-2X37S-BB Gyro Compass 3 x Sperry Marine NAVIGAT X MK1 Magnetic Compass SPERRY MARINE ECDIS FURUNO “TECDIS” DP system KONGSBERG K-POS DP21 Navigation Echo Sounder: FURUNO FE-720 GMDSS FURUNO A1, A2, A3 VDR FURUNO VR-3000 AIS FURUNO FA-150 Speed Log FURUNO DS-60 Facsimile FURUNO FAX-408 SEISMIC EQUIPMENT Energy Source 2 x LMF48/138-207 48 m3/min (1699 cfm) 3 x LMF31/138-207 31 m3/min (1099 cfm Source Controller Bigshot Source Elements Sercel G-Gun II Sub Arrays up to 10 Volume up to 8000 cu.in. Operating Pressure 138 bar / 2000 psi On-line Navigation System: Project specific Primary Navigation: Project specific GPS receiver: Project specific Secondary navigation: Project specific GPS receiver: Project specific Gun array tracking: RGPS Kongsberg Seatrack 330 Acoustic: Project specific USBL QC QUALITY CONTROL Navigation QC Teledyne Smart Source D QC Seismic Processing Open CPS | SPEED AND FUEL CONSUMPTION Cruising Speed: 10,0 knots / 16.0 m3 Max speed: 14,0 knots / 21.0 m3 DP operations: var. / 12.0 m3 CAPACITIES Fuel: 2980 M3 at 98% Fresh water: 664 M3 Sewage: 23.8 M3 Oil Residue: 20,8 M3 Fresh water generator: ENWA Reverse Osmosis: 25 m3 Sewage treatment plant: 75 persons x RWO WWT LC 06, 10.8 m3/hr Incinerator: Atlas 400 SL WS 180.000 kcal/h; max70 kg/h LIFE SAVING EQUIPMENT Life Rafts: 12 x 16, Viking Rescue Boat: 1 x 15 persons, Jiangyin Neptune Marine FRP BA-sets 4 x SCOTT Air Compressor: 1 x MCH-13/ET Line Thrower: 4 x Pains/Wessex Survival Suits: 64 x VIKING Life Vests: 70 Life buoys: 12 Gas Monitor: 3 x portable sets-Drager Medical Emergency: 1x Heart-Start AED, Resuscitator Galley System: 1x CO2 HP System Emergency Fire Pump: 1x 45 m3/h Fire Extinguisher: 104/ CO2, Powder/Foam CO2 System: 1x Fixed ER 1x Fixed Galley 1x Fixed EDG COMMUNICATION Inmarsat Fleet broadband: FELCOM 250 VSAT equipment: MARLINK Inmarsat C: FURUNO Felcom 15 Emergency Radios: 3x VHF ENTEL HT649 M/F and H/F: FURUNO FS-1570 VHF stationary: FURUNO FM-8800S Internal communication: 1 Automatic Telephone System “VINGTOR” 1 Battery less Telephone System “VINGTOR” 1 Public Address System Navtex: NX-700 Vessel E-mail: captain@andes.remoffshore.no ACCOMMODATION Accommodation capacity: 60 persons 1M Cabins 20 x 1M Cabins with Bathrooms 2M Cabins 20 x 2M Cabines with Bathrooms Mess: 42 person Day room: 1 x comfortable outfitted on Deck B Gym Area. Fully equipped HELIDECK D value 19.5 m “t” value 9.3 t Location Stern |
| --- | --- |